7 SEGMENT LED

/*

Pin Configuration

A		- 	P1.16

B		-	P1.17

C		-	P1.18

D		-	P1.19

E		-	P1.20

F		-	P1.21

G		-	P1.22

DOT		-	P1.23

SEG1	-	P0.10

SEG2	-	P0.11

*/

#include"lpc214x.h"

#define SEG_A  1<<16			   // SEGMENT 	A

#define SEG_B  1<<17			   // SEGMENT 	B

#define SEG_C  1<<18			   // SEGMENT 	C

#define SEG_D  1<<19			   // SEGMENT 	D

#define SEG_E  1<<20			   // SEGMENT 	E

#define SEG_F  1<<21			   // SEGMENT 	F

#define SEG_G  1<<22			   // SEGMENT 	G

#define SEG_DP 1<<23			   // SEGMENT 	DECIMAL POINT

#define SEG1   1<<10

#define SEG2   1<<11

#define ONE 	(SEG_B | SEG_C)	   										//DATA FOR SHOWING '1'

#define TWO 	(SEG_A | SEG_B | SEG_D | SEG_E | SEG_G)					//DATA FOR SHOWING '2'

#define THREE 	(SEG_A | SEG_B | SEG_C | SEG_D | SEG_G)					//DATA FOR SHOWING '3'

#define FOUR 	(SEG_B | SEG_C | SEG_F | SEG_G)							//DATA FOR SHOWING '4'

#define FIVE 	(SEG_A | SEG_C | SEG_D | SEG_F | SEG_G)					//DATA FOR SHOWING '5'

#define SIX 	(SEG_A | SEG_C | SEG_D | SEG_E | SEG_F | SEG_G)			//DATA FOR SHOWING '6'

#define SEVEN 	(SEG_A | SEG_B | SEG_C)	   								//DATA FOR SHOWING '7'

#define EIGHT 	(SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F | SEG_G)	//DATA FOR SHOWING '8'

#define NINE 	(SEG_A | SEG_B | SEG_C | SEG_D | SEG_F | SEG_G)			//DATA FOR SHOWING '9'

#define ZERO 	(SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F)			//DATA FOR SHOWING '0'

#define AA 		(SEG_A | SEG_B | SEG_C | SEG_E | SEG_F | SEG_G)			//DATA FOR SHOWING 'A'

#define BB 		(SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F | SEG_G)	//DATA FOR SHOWING 'B'

#define CC 		(SEG_A | SEG_D | SEG_E | SEG_F)							//DATA FOR SHOWING 'C'

#define DD 		(SEG_A | SEG_B | SEG_C | SEG_D | SEG_E | SEG_F )		//DATA FOR SHOWING 'D'

#define EE 		(SEG_A | SEG_D | SEG_E | SEG_F | SEG_G)			        //DATA FOR SHOWING 'E'

#define FF 		(SEG_A | SEG_E | SEG_F | SEG_G)					        //DATA FOR SHOWING 'F'

#define DP 		(SEG_DP)												//DATA FOR SHOWING '.'

#define SSEG_MASK   	(SEG_A|SEG_B|SEG_C|SEG_D|SEG_E|SEG_F|SEG_G|SEG_DP)

unsigned int seg_data[]={ZERO , ONE,TWO, THREE, FOUR, FIVE, SIX, SEVEN,EIGHT,\

NINE, AA, BB, CC, DD, EE, FF, DP };

void delay(unsigned int time)

{

unsigned int i,j;

for(i = 0; i < time ;i++ )

{

for(j = 0; j < 5000 ; j++);

}

}

int main(void)

{

int i,j;

PINSEL0 &= 0xFF0FFFFF;				//P0.10 and P.11 as GPIO

PINSEL2 = 0xFFFFFFF3;				//PORT1 as GPIO

IO0DIR = SEG1 | SEG2;

IO1DIR = SSEG_MASK;	// set the direction of the pins

IO1SET= SSEG_MASK;	// set the direction of the pins

while(1)

{

for(i =0 ; i < 99 ; i++ )

{

for(j=0; j<5; j++)

{

IO0SET = SEG1;

IO0CLR = SEG2;

IO1CLR = seg_data[i/10];	// since the 7 seg display is common anode we have to clear the port pins to make SEGMENT

delay(10);

IO1SET = seg_data[i/10];

IO0CLR = SEG1;

IO0SET = SEG2;

IO1CLR = seg_data[i%10];	// since the 7 seg display is common anode we have to clear the port pins to make SEGMENT

delay(10);

IO1SET = seg_data[i%10];

}

}

}

}