BLINKING OF LED’s

/********************************************************/

/* This Program is to demonstrate the LED Functionality */

/*******************************************************

Author 		: Ashish

Board 		: Micro_A748_SI

Processor 	: LPC2148

LED Mapping 	: LED0 thru LED7 : P1.24 thru P1.31

Jumper Settings :

********************************************************/

#include<lpc214x.h>

//LED Mapping for the board

#define LED_MASK 0xFF000000

#define LED0  24

void delay(unsigned int time)

{

unsigned int i,j;

for(i = 0; i < time ;i++ )

{

for(j = 0; j < 5000 ; j++);

}

}

int main(void)

{

//PINSEL2 &= 0xFFFFFFF3;	// set PORT1 as GPIO

PINSEL2 = 0;

IODIR1 = LED_MASK;			// set the direction of the pins as  output : 1

while(1)

{

IOSET1 = LED_MASK;					// set the port pins to 1

delay(1500);

IOCLR1 = LED_MASK;					//clear the port pins to 0

delay(1500);

}

}