Even LED BLINKING/CHESSBOARD PATTERN

/********************************************************/

/* This Program is to demonstrate the LED Functionality */

/*******************************************************

Author 		: Ashish

Board 		: Micro_A748_SI

Processor 	: LPC2148

LED Mapping 	: LED0 thru LED7 : P1.24 thru P1.31

Jumper Settings :

********************************************************/

#include <lpc214x.h>

/* -- Pin definitions --------------------------------------------- */

#define LED_MASK    0xFF000000   // P1.24 – P1.31  (all 8 LEDs)

#define LED_SHIFT   24           // LEDs start at bit 24

/*

LED positions:

Bit:  31   30   29   28   27   26   25   24

LED:  L7   L6   L5   L4   L3   L2   L1   L0

*/

/* Pattern definitions (bit0 = LED0 at P1.24, bit7 = LED7 at P1.31) */

#define EVEN_LEDS       0xAA000000   // LEDs 1,3,5,7  ? 1010 1010

#define ODD_LEDS        0x55000000   // LEDs 0,2,4,6  ? 0101 0101

#define CHESS_A         0xAA000000   // Chess pattern A ? 1010 1010

#define CHESS_B         0x55000000   // Chess pattern B ? 0101 0101

/* -- Delay ------------------------------------------------------- */

void delay(unsigned int time)

{

unsigned int i, j;

for(i = 0; i < time; i++)

for(j = 0; j < 5000; j++);

}

/* -- Main --------------------------------------------------------- */

int main(void)

{

PINSEL2 = 0;              // PORT1 as GPIO

IODIR1  = LED_MASK;       // P1.24–P1.31 as OUTPUT

while(1)

{

/* ====================================================

PART 1 : EVEN LED BLINKING

Only even-numbered LEDs (LED0, LED2, LED4, LED6) blink

Odd LEDs stay OFF

====================================================

LED:   7    6    5    4    3    2    1    0

EVEN:  OFF  ON   OFF  ON   OFF  ON   OFF  ON  = 0x55000000

*/

IOCLR1 = LED_MASK;         // turn ALL LEDs off first

delay(200);

IOSET1 = EVEN_LEDS;        // turn ON even LEDs (0,2,4,6)

delay(200);

IOCLR1 = EVEN_LEDS;        // turn OFF even LEDs

delay(200);

IOSET1 = EVEN_LEDS;        // blink again ON

delay(200);

IOCLR1 = EVEN_LEDS;        // blink again OFF

delay(200);

/* ====================================================

PART 2 : CHESS BOARD PATTERN

Alternates between two complementary patterns

like black and white squares on a chess board

Pattern A:  OFF ON  OFF ON  OFF ON  OFF ON  = 0xAA000000

L7  L6  L5  L4  L3  L2  L1  L0

Pattern B:  ON  OFF ON  OFF ON  OFF ON  OFF = 0x55000000

L7  L6  L5  L4  L3  L2  L1  L0

====================================================

*/

IOCLR1 = LED_MASK;         // clear all before chess pattern

delay(200);

IOSET1 = CHESS_A;          // Chess pattern A ? alternate ON/OFF

delay(200);

IOCLR1 = CHESS_A;          // clear pattern A

IOSET1 = CHESS_B;          // Chess pattern B ? inverse of A

delay(200);

IOCLR1 = CHESS_B;          // clear pattern B

IOSET1 = CHESS_A;          // back to pattern A

delay(200);

IOCLR1 = CHESS_A;          // clear

IOSET1 = CHESS_B;          // pattern B again

delay(200);

IOCLR1 = LED_MASK;         // clear all, cycle repeats

delay(200);

}

return 0;

}